PRICE

Office of the Chief Engineer LSGD

Leach pit – Household Greywater management

Detailed Estimate

(Cost Index Applied for this estimate is 35.59%)

| Sl No | Description | No | L | B | D | CF | Quantity | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| House hold Leach pit |  |  |  |  |  |  |  |  |
| 1 | 2.6.1 Earth work in excavation by mechanical means (Hydraulic excavator)/manual means over areas (exceeding 30 cm in depth, 1.5 m in width as well as 10 sqm on plan) including disposal of excavated earth, lead up to 50 m and lift up to 1.5 m, disposed earth to be leveled and neatly dressed. All kinds of soil |  |  |  |  |  |  |  |
|  | Leach pit | 1 | 3.14 | 0.745*0.745 | 1 |  | 1.743 |  |
|  | Total Quantity |  |  |  |  |  | 1.743 cum |  |
|  | Total Deducted Quantity |  |  |  |  |  | 0.000 cum |  |
|  | Net Total Quantity |  |  |  |  |  | 1.743 cum |  |
|  | Say 1.743 cum @ Rs 214.03 / cum |  |  |  |  |  | Rs 373.05 |  |
| 2 2. | 4.1.8 Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work up to plinth level: 1:3:6 (1 cement : 3 coarse sand : 6 graded stone aggregate 20 mm nominal size) |  |  |  |  |  |  |  |
|  |  | 1 | 3.14 | 0.745*0.745- 0.365*0.365 | 0.075 |  | 0.099 |  |
|  | Total Quantity |  |  |  |  |  | 0.099 cum |  |
|  | Total Deducted Quantity |  |  |  |  |  | 0.000 cum |  |
|  | Net Total Quantity |  |  |  |  |  | 0.099 cum |  |
|  | Say0.10 cum @ Rs 7211.15 / cum |  |  |  |  |  | Rs. 721.12 |  |
| 3 2. | 6.2.2 Brick work with common burnt clay modular bricks of class designation 7.5 in foundation and plinth in: Cement Mortar 1:6 (1 cement : 6 coarse sand) |  |  |  |  |  |  |  |
|  |  | 1 | 3.14 | 0.67*0.67- 0.44*0.44 | 0.075 |  | 0.060 |  |
|  |  | 1 | 3.14 | 0.61*0.61-0.50*0.50 | 0.85 |  | 0.329 |  |
|  | Honeycomb (LS) (- deduction) |  |  |  |  |  | -0.033 |  |
|  | Total Quantity |  |  |  |  |  | 0.389 cum |  |
|  | Total Deducted Quantity |  |  |  |  |  | 0.033 cum |  |
|  | Net Total Quantity |  |  |  |  |  | 0.356 cum |  |
|  | Say 0.356 cum @ Rs 6241.61/ cum |  |  |  |  |  | Rs. 2222.01 |  |
| 4 | 4.4.1 Providing and laying cement concrete in kerbs, steps and the like at or near ground level excluding the cost of centering, shuttering and finishing.1:1.5:3 (1 cement : 1.5 coarse sand (Zone III) : 3 graded stone aggregate 20 mm nominal size) |  |  |  |  |  |  |  |
|  | Bed Blocks | 1 | 3.14 | 0.61*0.61-0.50*0.50 | 0.075 |  | 0.034 |  |
|  | Cover Slab | 1 | 3.14 | 0.61*0.61 | 0.075 |  | 0.088 |  |
|  | Total Quantity |  |  |  |  |  | 0.122 cum |  |
|  | Total Deducted Quantity |  |  |  |  |  | 0.00 |  |
|  | Net Total Quantity |  |  |  |  |  | 0.122 cum |  |
|  | Say 0.122 cum @ Rs 8487.46/ cum |  |  |  |  |  | Rs. 1035.47 |  |
| 5 | 5.22.1 Steel reinforcement for R.C.C work including straightening, cutting, bending, placing in position and binding all complete upto plinth level. Mild steel and Medium Tensile steel bars |  |  |  |  |  |  |  |
|  | Cover Slab (as per 4.) | 1 | 0.088 |  |  | 100 | 8.8 |  |
|  | Total Quantity |  |  |  |  |  | 8.8 KG |  |
|  | Total Deducted Quantity |  |  |  |  |  | 0 |  |
|  | Net Total Quantity |  |  |  |  |  | 8.8 KG |  |
|  | Say 10KG @ Rs 96.68/ KG |  |  |  |  |  | Rs. 966.80 |  |
| 6 | 50.18.9.8.2 Providing and fixing PVC pipes includings jointing of pipes with one step PVC solvent cement, trenching, refilling & testing of joints complete as per direction of Engineer in Charge. 75 mm dia 4 Kgf/ cm2 |  |  |  |  |  |  |  |
|  | PVC Pipe | 1 | 1.5 | 0 | 0 |  | 1.5 |  |
|  | Total Quantity |  |  |  |  |  | 1.5 m |  |
|  | Total Deducted Quantity |  |  |  |  |  | 0 |  |
|  | Net Total Quantity |  |  |  |  |  | 1.5 m |  |
|  | Say 1.5 @ Rs 318.89 /m |  |  |  |  |  | Rs 478.33 |  |
|  | Amount Reserved for GST Payments |  |  |  |  |  |  | 1043.42 |
|  | Sub Total |  |  |  |  |  |  | 6,840.21 |
|  | Lump Sum Round Off |  |  |  |  |  |  | 59.79 |
|  | Total |  |  |  |  |  |  | 6900 |
|  | Six thousand and Nine Hundred rupees only |  |  |  |  |  |  |  |